CMSC 313 Project 2

Name:

Assigned: 10/6/2026

Due: 10/27/2026 at 11:59pm

Project Description
In this project, you will design several components of a 4-bit multi-purpose processor using Logisim Evolution. A skeleton file named Project2_Skeleton.circ will help you get started – make sure to download it from Blackboard. Read these directions carefully before starting.

Please rename the skeleton file to Project2_YOURLASTNAME.circ before submitting the project to Blackboard. Do NOT change any of the subcircuit names in Logisim Evolution. Do NOT add, remove, or modify the properties of any input or output pins. If you move any input or output pins, you must keep them in the same vertical ordering. If you do not follow these instructions, you may have up to 50% of your project grade deducted.

Update Logisim to version 5.0.0, if you are not currently on that version:

- https://github.com/logisim-evolution/logisim-evolution/releases

Given Sub-Circuits
You have been given several working sub-circuits in the Skeleton file. Do NOT change any of these sub-circuits.

Incrementor – An Incrementor component.

- Incrementor inputs:

  - A – A 4-bit integer

- Incrementor outputs:

  - Q – A 4-bit integer that is equal to A3A2A1A0 plus 1. If A3A2A1A0=1111, the incrementor overflows and Q3Q2Q1Q0=0000.

Arithmetic_Logic_Unit – An ALU that is behaviorally identical to Project 1.

- ALU Inputs:

  - A – A 4-bit data input

  - B – A second 4-bit data input

  - Opcode – A three-bit input (O2O1O0) that determines the ALU’s operation.

- ALU Outputs:

  - Q – The 4-bit output from the ALU.

  - Z_flag – The Zero flag is 1 when Q3Q2Q1Q0=0000, and 0 otherwise.

ALU Operations:

| Opcode | Operation |
| --- | --- |
| 000 | Q = B (No_Operation / Move) |
| 001 | Q = A == B (Equality_Comparison) |
| 010 | Q = A XOR B (Bitwise_XOR) |
| 011 | Q = A (Inverter) |
| 100 | Q = A + 1 (Increment) |
| 101 | Q = A + B (Addition) |
| 110 | Q = A - B (Subtraction) |
| 111 | Q = A * B (Multiplication) |

Register_File – A file of four general-purpose registers that is almost behaviorally identical to HW 3.

- Register_File inputs:

  - D – the 4-bit data input.

  - WE – the Write-Enable input. If it is 1, then exactly 1 of the registers should be write-enabled. If it is 0, then none of the registers should be write-enabled.

  - W_id – the 2-bit ID of the register that should be write-enabled, if WE is 1.

  - R_id_A – the 2-bit ID of a register. The value of the register with ID R_id_A should be stored in Q_A.

  - R_id_B – the 2-bit ID of another register. The value of the register with ID R_id_B should be stored in Q_B.

  - Clk is the clock input.

  - Reset is the reset input. If Reset=1, all registers become 0.

- Register_File outputs:

  - Q_A – the value stored in the register with the ID R_id_A.

  - Q_B – the value stored in the register with the ID R_id_B.

Flag_Register – Normally, an ALU has flag outputs describing the result of its last operation (such as if the result was 0, if it was negative, if an overflow happened, etc). These flags are stored together in a single register. For simplicity, the flag register will only store one flag, named Z_Flag.

- Flag_Register inputs:

  - Z_in – the zero flag to store in the flag register. Its value is 1 if the ALU’s output was equal to zero, and its value is 0 if the ALU’s output was non-zero.

  - WE – the write-enable bit. Instructions are either ALU instructions or Jump instructions. All ALU instructions modify one of the registers in the register file, so WE=1 for these instructions. Jump instructions do not affect the register file, so WE=0 for them. If WE is 0, the flag register does not update.

  - Clk – the clock input.

  - Reset – the reset input. If Reset=1, the flag register becomes 0.

- Flag_Register outputs:

  - Z_flag: The value of the zero flag currently stored in the flag register.

Part 1: PC_Register (25 pts)
The Program Counter (PC) register is a component that stores the address of the instruction to execute next. The ROM component in the Multi_Purpose_Processor has 16 9-bit memory entries. The first memory entry has the address 0, the second memory entry has the address 1, and so on. (The sixteenth memory entry has the address 0xF, since the built-in Logisim ROM and registers show their values in hexadecimal).

- PC_Register inputs:

  - Do_Jump – If this bit is 1, the Program Counter’s register should become equal to Jump_Addr. If this bit is 0, the Program Counter’s register should increment instead.

  - Jump_Addr – The 4-bit address in the Multi_Purpose_Processor’s ROM component to “jump to”. If Do_Jump is 1, the program counter’s register should become equal to Jump_Addr.

  - Clk – The clock cycle input.

  - Reset – When this input is 1, the Program Counter’s register should become 0.

- PC_Register outputs:

  - M_addr  – The 4-bit address of the machine code (in ROM) that should be executed next by the Multi_Purpose_Processor.

When the program begins, the PC register should be initialized to 0, meaning that the instruction at address 0 will be executed first. At the rising edge of each clock cycle, if the Do_Jump input is 0, the Program Counter’s register should increase by 1 using the given Incrementor component. If the Do_Jump input is 1, the Program Counter’s register should become equal to the J_addr input, rather than incrementing.

In the skeleton file, a Multiplexor, an Incrementor, and a Logisim register have been placed at the bottom of the circuit. They will each be used somewhere in PC_Register.

Part 2: Control_Unit (40 pts)
The Control Unit is responsible for interpreting machine code during the “Decode” phase of the Fetch/Decode/Execute cycle. Each machine code instruction is a 9-bit value, represented as 3-digit hexadecimal values in the Multi_Purpose_Processor’s ROM component. For example, the hexadecimal value 0x1A7 is equivalent to the 9-bit machine code 1 1010 0111.

- The inputs to the Control Unit are:

  - M – The 9-bit machine code instruction.

  - Z_flag – The value of the zero flag.

A splitter has been given to you, to separate M into its 9 bits (M8M7M6M5M4M3M2M1M0). M0 is the wire at the top and M8 is the wire at the bottom.

M for non-Jump instructions:

| M8 | M7 | M6M5M4 | M3M2 | M1M0 |
| --- | --- | --- | --- | --- |
| Jump_instr | Operand | Opcode | Dst ID | Src ID |

M for Jump instructions:

| M8 | M7 | M6M5M4 | M3M2M1M0 |
| --- | --- | --- | --- |
| Jump_instr | Operand | Opcode | Jump_Addr |

- Control Unit outputs:

  - WE – the write-enable bit. Instructions are either ALU instructions or Jump instructions. All ALU instructions modify one of the registers in the register file, so WE=1 for these instructions. Jump instructions do not affect the register file, so WE=0 for them. If WE is 0, the flag register does not update.

  - W_id – the 2-bit ID of the register that should be write-enabled, if WE is 1.

  - R_id_A – the 2-bit ID of a register. The value of the register with ID R_id_A should be stored in Q_A.

  - R_id_B – the 2-bit ID of another register. The value of the register with ID R_id_B should be stored in Q_B.

  - Opcode – The 3-bit opcode indicating which operation the ALU should perform.

  - Operand – If this value is 1, a multiplexor in the Multi_Purpose_Processor will forward I to the B input of the ALU. If it is 0, it will forward Q_B from the register file to the B input of the ALU.

  - Do_Jump – Whether or not the current instruction should perform a jump. This depends on whether the current instruction is a jump instruction, the opcode, and possibly the state of Z_flag.

  - Jump_Addr – The 4-bit address to jump to.

Instruction Set Architecture
Below is a table of the Multi_Purpose_Processor’s Instruction Set Architecture

- It shows the upper 5 bits of M, the assembly instruction, the instruction’s behavior, and whether or not the instruction affects the Z_flag.

- dst denotes the destination register (W_id and R_id_A) and src denotes the source register (R_id_B).

- The symbol “:=” is the assignment operator.

| Jump_instr (M8) | Operand (M7) | Opcode (M6M5M4) | Assembly Instruction | Instruction Behavior | Z_Flag Affected? |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 000 | MOV dst, src | dst := src | Yes |
| 0 | 0 | 001 | TEST dst, src | dst := dst == src | Yes |
| 0 | 0 | 010 | XOR dst, src | dst := dst ^ src | Yes |
| 0 | 0 | 011 | NOT dst | dst := ~dst | Yes |
| 0 | 0 | 100 | INC dst | dst := dst + 1 | Yes |
| 0 | 0 | 101 | ADD dst, src | dst := dst + src | Yes |
| 0 | 0 | 110 | SUB dst, src | dst := dst – src | Yes |
| 0 | 0 | 111 | MUL dst, src | dst := dst * src | Yes |
| 0 | 1 | 000 | MOV dst, I | dst := I | Yes |
| 0 | 1 | 001 | TEST dst, I | dst := dst == I | Yes |
| 0 | 1 | 010 | XOR dst, I | dst := dst ^ I | Yes |
| 0 | 1 | 011 | NOT dst | dst := ~dst | Yes |
| 0 | 1 | 100 | INC dst | dst := dst + 1 | Yes |
| 0 | 1 | 101 | ADD dst, I | dst := dst + I | Yes |
| 0 | 1 | 110 | SUB dst, I | dst := dst – I | Yes |
| 0 | 1 | 111 | MUL dst, I | dst := dst * I | Yes |
| 1 | Either | 000 | JZ J_addr | PC := Jump_Addr if Z_flag == 1 | No |
| 1 | Either | 001 | JNZ J_addr | PC := Jump_Addr if Z_flag == 0 | No |
| 1 | Either | 010 | JMP J_addr | PC := Jump_Addr | No |
| 1 | Either | 011 | Not Used | N/A | N/A |
| 1 | Either | 100 | Not Used | N/A | N/A |
| 1 | Either | 101 | Not Used | N/A | N/A |
| 1 | Either | 110 | Not Used | N/A | N/A |
| 1 | Either | 111 | Not Used | N/A | N/A |

Instructions for Wiring the Control Unit

You may use any basic logic gates (AND, OR, NOT, etc.) and splitters in the Control Unit. You may not use any other components. You may configure your basic logic gates so that they have more than 2 inputs.

The lower 4 bits of M are used for two different purposes. If the current instruction is a Jump instruction, then M3M2M1M0 is the 4-bit address that the program intends to jump to. If the current instruction is not a Jump instruction, then M3M2 is used for W_id and R_id_A, and M1M0 is used for R_id_B.

- WE – This should be 0 if the current instruction is a Jump instruction, and 1 if the current instruction is not a Jump instruction. The M8 bit of the machine code determines whether or not the current instruction is a Jump instruction or not.

- W_id – This two-bit value should be wired directly to M3M2. Make sure that the upper bit of W_id connects to M3 and the lower bit of W_id connects to M2.

- R_id_A – This two-bit value should be wired to M3M2.

- R_id_B – This two-bit value should be wired to M1M0.

- Opcode – This 3-bit value should be wired to M6M5M4.

- Operand – This 1-bit value should be wired to M7.

- Do_Jump – This 1-bit value indicates whether a Jump should be taken. More directions below.

- Jump_Addr – This 4-bit value should be wired to M3M2M1M0.

The Do_Jump output is a bit more complicated than the rest. It depends on several things. First, Do_Jump can only be 1 if the instruction is a jump instruction (M8 = 1). It also depends on the type of Jump instruction, and potentially the value of Z_flag.

- JZ – If M8 is 1,  Opcode (M6M5M4) is 000 and Z_flag is 1, then Do_Jump should be 1.

- JNZ - If M8 is 1,  Opcode (M6M5M4) is 001 and Z_flag is 0, then Do_Jump should be 1.

- JMP - If M8 is 1,  Opcode (M6M5M4) is 010, then Do_Jump should be 1.

- Do_Jump should be 0 for all other values of M.

Machine Code Example
Here is an example of an assembly instruction and its machine code representation:

- Assembly: ADD Reg_00, Reg_11

- Machine code: 0 0101 0011

- Operation: Reg_00 := Reg_00 + Reg_11

The 9 bit machine code M from the example is broken down into the following parts:

- M1M0 depend on whether this is an ALU instruction or a Jump instruction.

  - When Jump_instr == 0,  M1M0 become R_id_B. In this example, this means to read from the register with address 11.

  - When Jump_instr == 1, M1M0 are the lower two bits of Jump_Addr, the 4-bit address to jump to.

- M3M2 also depend on whether this is an ALU instruction or a Jump instruction.

  - When Jump_instr == 0, M3M2 become W_id and R_id_A. In this example, this means to read from the register with address 00 and store the result in the register with address 00.

  - When Jump_instr == 1, M3M2 are the higher two bits of Jump_Addr, the 4-bit address to jump to.

- M6M5M4 are the opcode for the ALU. The opcode controls which operation the ALU executes. In this example, M6M5M4 are 101, which means that the ALU performs addition.

- M7 is the Operand bit. Since it is 0, the B input of the ALU comes from Q_B, rather than I.

- M8 is the Jump_instr bit. Since it is 0, this is not a Jump instruction.

More Machine Code Examples
Example 1:

- Assembly: ADD Reg_01, I

- Machine code: 0 1101 01XX

- Behavior: Add the input I to Reg_01, and store the result in Reg_01. M1M0 does not matter in this example, and can have any value.

Example 2:

- Assembly: INC Reg_10

- Machine code: 0 X100 10XX

- Increment the contents of Reg_10 by 1 and store the result in Reg_10. M7 and M1M0 do not matter in this example, and can have any value.

Example 3:

- Assembly: JZ 6

- 1 X000 0110

- Jump to the machine code at address 6 if Z_flag is equal to 1. M7 does not matter in this example. M3M2M1M0 (which are normally used as register IDs in ALU instructions) are equal to 6, which is the address that will be jumped to.

Part 3: Multi_Purpose_Processor (35 pts)

- Multi_Purpose_Processor inputs:

  - I – a 4-bit input to the ALU.

  - Clk – a Logisim clock component.

  - Reset – the reset input. If Reset=1, all registers become 0.

- The outputs of the multi-purpose processor are:

  - Q_A – a 4-bit value from the read port of the register file.

  - Q_B – another 4-bit value from the read port of the register file.

A multiplexor and a ROM component have been placed at the bottom of the Multi_Purpose_Processor. You should use them somewhere in the Multi_Purpose_Processor. Place an Arithmetic_Logic_Unit, Register_File, Flag_Register, PC_Register, and Control_Unit into the Multi_Purpose_Processor. Then, wire together a working Multi_Purpose_Processor.

The ALU’s A input should receive the Q_A output of the Register_File. The ALU’s B input should receive either the Register_File’s Q_B output, or I, depending on the value of Operand.

You may not use logic gates, splitters, or any other components in Logisim to complete the Multi_Purpose_Processor, except for wires and the components listed above.

PC_Register Testing
In the top bar in Logisim-Evolution, click Simulate -> Reset Simulation. Then, navigate to the PC_Register component.

- Click the Reset input twice. The value of the register should be 0, and the value of M_Addr should be 0000.

- Click the Clk input eight times. Each time Clk is 1, the register value and M_Addr should increase by 1. After doing this, the value of the register should be 4, and M_addr should be 0100.

- Set Do_Jump to 1 and set Jump_Addr to 1101. Then, click Clk twice. The value of the register should be “d” (13 in hexadecimal) and M_addr should be 1101.

Control Unit Testing
In the top bar in Logisim-Evolution, click Simulate -> Reset Simulation. Navigate to the Control_Unit.

- Select the M input pin. On the bottom left window, scroll down to “Radix” and set it to Binary. Set the following values:

  - M = 1 0000 1000

  - Z_flag = 1

- After step 1, the output pins of the Control Unit should be:

  - WE=0

  - W_id=10

  - R_id_A = 10

  - R_id_B = 00

  - Opcode = 000

  - Operand = 0

  - Do_Jump = 1

  - Jump_Addr = 1000

- Next, set these input values:

  - M = 0 1101 1110

  - Z_flag = 0

- After step 3, the output pins of the control unit should be:

  - WE=1

  - W_id=11

  - R_id_A = 11

  - R_id_B = 10

  - Opcode = 101

  - Operand = 1

  - Do_Jump = 0

  - Jump_Addr = 1110

Multi_Purpose_Processor Testing
In the top bar in Logisim-Evolution, click Simulate -> Reset Simulation. Navigate to the Multi_Purpose_Processor, then follow these instructions.

- Right-click the ROM component and select “Edit Contents”. Enter the following:

  - 080 040 004 0E4 116 088 126 (leaving the rest of the entries on the right as 000)

- Set the value of I to 0011.

- Repeatedly click the Clk twice to simulate a clock cycle. After each clock cycle, check the results of your program below. If your Multi_Purpose_Processor is wired correctly, you should see these results:

  - After the first clock cycle, Q_A and Q_B are 0011

  - After the second clock cycle, Q_A is 0000 and Q_B is 0100

  - After the third clock cycle, Q_A and Q_B are 0100

  - After the fourth clock cycle, Q_A is 0001 and Q_B is 0000

  - After the fifth clock cycle, the value 126 is highlighted in the ROM, Q_A is 0001 and Q_B is 0000.

  - Cycling the clock after that does nothing.

These tests are not comprehensive, and you are encouraged to test each component more thoroughly.

Grading Breakdown

- 25 points: Program Counter works correctly.

- 40 points: Control Unit works correctly.

- 35 points: Multi_Purpose_Processor works correctly.

Partial credit will be given for circuits whose designs are nearly correct but have minor errors.

How to Submit

Test your circuit to make sure that everything works properly. Make sure your file is named Project2_YOURLASTNAME.circ. Submit your completed .circ file on Blackboard.